Advanced Scientific Concepts 10

Procedure Sheet

Graphing Your Motion

Clamp motion detector to surface.  Make sure a cone shaped area in front of it is free of objects and people.

- Connect motion detector to the CBL II (plug into the sonic port).  Connect the calculator and CBL II.

- Turn on the calculator (lower left corner) and select the “apps” key (it is blue).

- Select 3 “datamate”.

- The next screen that appears will be the home screen (it has 6 options on the bottom).  Press the “clear” key and the calculator should go to “checking sensors.”

- The top of the screen should read “dig: motions (N)”

- When you are ready to start have a student stand in front of the detector (at least ½ of a meter away but not more than 11 meters away).

- The student in front of the motion detector will swing their hips to create a transverse wave.  That person will want to hold a book or folder on their side for the ultrasonic wave to bounce off.

- A second student will select start when you are ready.  There will be a beeping and then a clicking.  When the motion detector is clicking, it is collecting data.  When it is done, it will beep again.

- You now have to select what type of graph you would like to see.  Make sure the arrow is pointing to the one you would like to view.   Select 3 “rescale”.

- When the graph appears, press “enter”.  Select 3 “y scale”.

- FOR A DISTANCE GRAPH:

- The word ymin= ? will appear.  Press -.5 and then “enter”.  For y max press 8 and then “enter”.  For yscl press .5 and then “enter”.  Your graph should appear (more “cleaned” up).

- FOR A VELOCITY GRAPH:

- The word ymin= ? will appear.  Press -5 and then “enter”.  For y max press 15 and then “enter”.  For yscl press 1 and then “enter”.  Your graph should appear (more “cleaned” up).

Press “enter”, then press 4 “return”, then press 1 “main screen”.  And you are ready to start again or select 6 “quit”.