| Smart Irrigation System: Challenge #2 | Goals: |
| --- | --- |
| In this activity you will use a Soil Moisture Sensor to read and report the soil moisture level. You will use the measured moisture level to display if the soil is dry or moist. | - Use the read command to read moisture levels - Use the range command to scale values - Use a For loop to repeat code. |
| Background: |  |
| Humans invented agriculture over 6000 years ago as a way to produce more food than could be hunted and gathered from the environment. This increase in food availability produced a rapid increase in population that was totally dependent on agriculture for survival. Today, the world’s large population requires sophisticated large-scale agriculture to keep everyone fed. Climate changes can have severe consequences on the food production required to keep the large human populations fed. Natural and man-made ecological disasters, such as the Dust Bowl of the 1930’s, can have severe consequences on the dependent populations. A more recent example, told in April 2016 The Guardian article*, describes the devastating effects on the local population of a severe drought in Zimbabwe. Science and technology can help to optimize food production and mitigate the effects of climate change and poor farming practices Command; Example; Behavior. CONNECT <type> <number> TO <port>; Send “CONNECT LIGHTLEVEL 1 TO IN1”; Associates the first LIGHTLEVEL object with a light sensor plugged into port IN1 on the Hub.. SET <type> <number> TO <value>; Send “SET ANALOG.OUT 1 TO 128”; Turns on an analog.out1 object, such as a pump, to a power setting of 128. RANGE <type> <number> <min value> <max value>; Send “RANGE LIGHTLEVEL 1 0 100”; Scales the measured values read from LIGHTLEVEL 1 to return in the range 0 to 100.. READ <type> <number>; Send “READ MOISTURE 1”; Reads one measurement from the first moisture sensor.. Get <variable>; Get m; Stores the moisture measurement into the variable named m. *Note a get command must immediately follow a read command. The value stored will contain the measurement from the immediately preceding READ command.”. DispAt <line #> , <”text”> , <variable name>; DispAt 3, “Moisture level = ”, m; When variable m has a value of 26, “Moisture level = 26” is displayed on line 3 of the calculator.. For <var>,<start>,<stop> <statements> EndFor; For count, 1, 5 Send “READ MOISTURE 1” Get m DispAt 3,”Moisture level = “,m Wait 1 EndFor; The commands inside the For structure will execute 5 times. The variable count is initially assigned a value of 1 then increases by 1 each time through the loop.. If <Boolean expression 1> Then <statements 1> ElseIf <Boolean expression 2> Then <statements 2> Else <statements 3> EndIf; If t >=40 Then DispAt 3,”It is Hot” ElseIf t >=25 and t < 40 Then DispAt 3,”It is Warm” Else DispAt 3,”It is Cool” EndIf; If the first Boolean expression is true, the corresponding statements are executed and the decision tree is immediately exited. In the example, if t=30 then the first expression is false and the <statements 1> are skipped, the second expression is true and <statements 2> are executed and the tree is exited. Additional ElseIf statements may be inserted if needed.. |
| The soil moisture sensor produces an output from 0 to 214 based on the conductivity (dielectric permittivity) of the soil which is dependent on water content. The sensor output is proportional with the volumetric water content; however, it is not calibrated in a particular unit or percentage. Like the light level sensor, it is useful to use the RANGE function to scale the sensor output from 0 to 100. Water interacts differently with different soil types. Fill in the missing information on the table below. Clay; Sand; Loam. Infiltration Rate. Water Retention. Texture. Erosion. |

Challenge:

Write a program named c2 that measures soil moisture every two seconds for a total of twenty times. The program should display if the soil is dry or moist based on the sensor reading.. Explore: - If permitted by your teacher, choose three different locations to measure. Record your results below. Condition: ________________________ Soil Moisture Level __________ Condition: ________________________ Soil Moisture Level __________ Condition: ________________________ Soil Moisture Level __________ - Give a range of reading levels you think would be ideal for growing crops. Why did you choose this range? - Do you think the ideal moisture levels change based on the plants growing in the soil? Can you give some examples?.