| Unit 4: Make Rover Move! | Application: Polygons |  |
| --- | --- | --- |
| The application for this unit is to program the Rover to make a polygon where the user enters the length of a side (in ‘Rover units’) and the number of sides of the polygon. The Rover will then make the polygon. | Objectives: |  |
|  | - Input statements - Using eval( ) in Rover instructions - Understanding polygon principles - Working with COLOR and timing (Wait) |  |
| Recall that the purpose of eval( ) is to convert the value of a handheld variable or expression into a string to be sent to the TI-Innovator™ Hub. In the sound program shown at the right, the user enters a frequency for the variable frequenc. The eval(frequenc) function converts that number into a string representation that the TI-Innovator Hub can process. |  |  |
| Write a program that asks for the length of a side and the number of sides of a polygon and moves the Rover in that pattern. You can add a marker to the Rover and actually draw the polygon on a large sheet of paper. Remember that 1 unit is 10 cm. Optional: Make the COLOR LED light up at the vertices of the polygon. |  |  |
| Some useful commands: Request “LENGTH OF SIDE?”,L Request “NUMBER OF SIDES?”,N For I,1,N Send “RV FORWARD eval(L)” Send “RV RIGHT <something>” EndFor |  |  |