| Unit 4: Make Rover Move! | Skill Builder 2: Driving Features |  |
| --- | --- | --- |
| In this lesson, you will investigate some of the optional driving features. The four driving commands in Skill Builder 1 all left a space character after the driving command inside the quotation marks. This is because there are other options that you can supply to the commands. This lesson investigates those options. | Objectives: |  |
|  | - Extending the driving features of the Rover - RIGHT and LEFT options - Wait |  |
| Parameters for FORWARD and BACKWARD These driving commands have three optional parameters: - SPEED - TIME - DISTANCE They are found in the menu > Hub > Rover (RV) > RV Settings menu partially shown at the right. Also, SPEED rates (UNITS/S and M/S) can also be accessed in this menu. |  |  |
| Using DISTANCE, SPEED and TIME Some examples of different implementations of the FORWARD command: - FORWARD DISTANCE # is the same as FORWARD # - FORWARD DISTANCE # M moves Rover # Meters - FORWARD # SPEED # where speed is between 1.4 and 2.3. - Values outside this range result in a TI-Innovator™ Hub error. - FORWARD TIME # You can specify any two of these three options in the FORWARD and BACKWARD commands. You can also use eval( ) if the value you want to use is stored in a handheld variable or if you want to use the result of an expression. |  |  |
| SPEED and TIME Program - The program shown at the right has a command that moves the Rover FORWARD for a certain SPEED and TIME: Send “RV FORWARD SPEED 2.3 TIME 2” - Complete the program by providing the correct TIME so that the Rover returns to its starting position: Send “RV BACKWARD SPEED 1.4 TIME ?” Hint: DISTANCE = SPEED * TIME |  |  |
| RIGHT and LEFT Options These commands by default turn the Rover RIGHT or LEFT 90 degrees but you can add a degree measure to the command to turn any angle (-360…360 degrees). Negative values are also permitted, so LEFT -90 is the same as RIGHT 90. - Add a command to make Rover turn RIGHT 135 degrees. You have to key in the 135 inside the closing quotation mark. The word DEGREES is not needed but is available in the RV Settings menu for clarity. You could also specify the angle measurement in RADIANS or GRADS but these units must be stated and are also found in the RV Settings menu. Some examples are shown to the right. What is the final heading of the Rover after these three statements are processed? |  |  |
| Write a program to make the Rover drive along the path of an equilateral triangle. Start with: Send “CONNECT RV” Send “RV FORWARD ?” Send “RV LEFT ?” Or, use a loop. |  |  |