| Unit 5: Rover’s Sensors | Skill Builder 2: Ranger and Movement |  |  |
| --- | --- | --- | --- |
| In the first lesson of Unit 5, you tested the Rover’s Ranger to see how to read the sensor and display a value. This lesson advances that testing to control the Rover’s motion. | Objectives: |  |  |
|  | - Use the READ RV.RANGER command to determine the distance to an obstacle - Control the Rover’s movement when the Rover gets too close to an obstacle. - Manage the timing of the Rover’s movement within the handheld program. |  |  |
| Let’s write a program to make the Rover move back and forth between two ‘walls’. We’ll start the Rover moving FORWARD, read the Ranger sensor and, when the Rover gets too close to the wall, make the Rover stop, turn around, and move FORWARD. |  |  |  |
| The Big Idea In a For loop (which will eventually terminate) Start the Rover moving FORWARD While the distance is greater than about 25 cm continue to monitor the Ranger sensor End the While loop STOP, turn RIGHT 180 End the For loop |  |  |  |
| - Begin the program in the usual manner. - Add a For loop that will iterate ten times. - Add a FORWARD 100 UNITS command to move ahead 10 meters (100 * 0.1m per unit) |  |  |  |
| Teacher Tip: There’s more code to go into the For loop body. At this point in the program, it appears that the Rover will move FORWARD 100 ten times. This is not the case in the final program because the Ranger will cause the Rover to stop and turn around. The FORWARD command goes into the ‘drive queue’ in the TI-Innovator™ Hub and the TI-Innovator Hub begins processing this queue immediately. |  |  |  |
| As in the previous lesson, we’ll use the variable dist to represent distance from the Ranger to an obstacle. - Initialize dist to 1, and insert a While dist > 0.25 loop. Note the two Ends in the code: an EndFor and an EndWhile. This is called ‘nested loops’. |  |  |  |
| - We next insert the code into the body of the While loop. - Add the Send “READ RV.RANGER” command. - Use the Get command to store the value into the variable dist. |  |  |  |
| That completes the While loop. The Rover is moving FORWARD 10 meters and the While loop is monitoring the distance. If desired, add a DispAt statement to this While loop to display the current distance to make sure things are working properly. When the While loop ends, this indicates that the Rover is too close to an obstacle. We’ll tell the Rover to STOP moving FORWARD and turn around. However, we do not need to tell Rover to move FORWARD again. |  |  |  |
| - After the End of the While loop but before the End of the For loop, add the RV STOP command and the RV RIGHT 180 command. - Add the Wait 2 statement to give the Rover time to turn around before moving FORWARD again. (Remember that the FORWARD command is at the beginning of the For loop.) - Test your program. When the Rover gets close to an obstacle, it should turn around and go in the opposite direction. Adjust values (minimum distance and Wait time), as needed, for your surface situation. If the Rover gets too close to the wall, then the back of the Rover could bump the wall when turning around. |  |  |  |
| Teacher Tip: A complete program (indenting for clarity only): Send "CONNECT RV" For i,1,10 Send("RV FORWARD 100") 1→d While d>0.25 Send "READ RV.RANGER" Get d EndWhile Send "RV STOP " Send "RV RIGHT 180" Wait 2 EndFor |  |  |  |