| Unit 5: Rover’s Sensors | Skill Builder 3: The COLORINPUT sensor |  |
| --- | --- | --- |
| In this third lesson of Unit 5, we introduce the COLORINPUT sensor and use its value to cause the Rover to change direction. | Objectives: |  |
|  | - READ COLORINPUT - Use the value to change the Rover’s direction |  |
| The Rover has a COLORINPUT sensor on the bottom. The light shining at the floor is used by the sensor to better ‘see’ the color on the floor. We’ll write a program to turn Rover when it ‘sees’ a change in color. The color seen by the sensor is converted by the TI-Innovator™ Hub into one of nine possible values that represent colors: 1 = Red 2 = Green 3 = Blue 4 = Cyan 5 = Magenta 6 = Yellow 7 = Black 8 = White 9 = Gray |  |  |
| - Begin the program in the usual manner. |  |  |
| - We’ll use a For loop to cause the Rover to move and turn four times. |  |  |
| - We will use a While loop to look for a change in color but first we need to know what color the Rover currently ‘sees’. We READ RV.COLORINPUT and Get its value into the variable floor_color in the handheld. |  |  |
| - We then initialize another variable, color, to get the While loop started. color will hold the color the Rover sees while it is moving. Initially, we set color to be the same as floor_color. Then, we start the Rover moving FORWARD. |  |  |
| - We now code the While loop to compare color with floor_color because, inside the While loop, we’ll look for a change in the value of color. - Don’t forget to add an EndWhile statement for the While loop. |  |  |
| - Inside the While loop, we monitor the color sensor as long as the Rover is moving. We store the color value in our variable color. The loop ends when color (the ‘current’ color) is different from floor_color (the ‘original’ color). |  |  |
| - At the end of the While loop, we tell the Rover to STOP and go RIGHT. Notice that these two statements fall between the EndWhile and the EndFor statements so the program causes the Rover to turn right four times when it detects a change in color of the surface. Note: “RV STOP” cancels all drive commands. - Add a FORWARD 1 command and a Wait 1 statement so that the Rover moves away from the colored spot before the program loops back to detect the new ‘original’ color (floor_color) again. - Test your program on the floor on a surface of uniform color (white). Place an approximately 5 cm by 5 cm contrasting color (black) spot on the floor in the Rover’s path so that the COLORINPUT sensor will see it. When the Rover passes over the spot, it should stop, and then turn and go again. Pick up the color spot, and place it in front or the Rover again. This will have to happen four times for the program to complete. |  |  |
| Teacher Tip: The completed program: |  |  |
| Teacher Tip: It’s possible that small variations in the surface might cause the Rover to turn unexpectedly. Even a line between tiles on a floor can result in a color change. The COLORINPUT sensor is very sensitive. |  |  |