| Unit 6: Coordinates | Skill Builder 1: Introduction to Coordinates |  |
| --- | --- | --- |
| In this first lesson for Unit 6, you will learn about the TI-Innovator Rover’s coordinate system and movement to coordinates. | Objectives: |  |
|  | - Understand the Rover coordinate system and initial position and heading - Make the Rover move to a certain point on the coordinate plane - Use mathematics to determine distance |  |
| The Rover has a ‘built-in’ coordinate system just like a graphing system. When you Send “CONNECT RV”, the Rover’s position on the coordinate grid is set to (0,0) and its heading is 0 degrees (points toward the positive x-axis). FORWARD moves the Rover to the right. LEFT turns the Rover 90 degrees counterclockwise. |  |  |
| Our program will tell the Rover to move to a point on its coordinate grid. We’ll use Request statements to enter values for x and y and then make the Rover drive to the point (x, y) and then move back to the origin. - Start your program with the usual instructions. - Include Request statements for x and y. Recall that Request will display the message entered in quotes (the ‘prompt’). - The Text statement gives you time to place the Rover at the origin and face the Rover in the ‘right’ direction. |  |  |
| - Add the Rover command to drive TO XY. The command is found in the menu > Hub > Rover (RV) > Drive RV menu. and will appear in your program as an incomplete statement. Send “RV TO XY ” The command will appear in your program as an incomplete statement. |  |  |
| - The x- and y-coordinates must be added and will be stored in the variables x and y, respectively. In order for the TI-Innovator™ Hub to use these values, you must use the eval( ) function twice. - Add eval(x) eval(y) to the command. Send “RV TO XY eval(x) eval(y)” - Test your program now. Depending on the coordinate values you enter, the Rover will move to that position. |  |  |
| - Add a Wait command to let the Rover move to your point, and then tell the Rover to return to the origin. Simply use the numbers 0 and 0 separated by a space. We also add a statement to make the Rover point in its original direction (TO ANGLE 0). - Test your program again. This time, the Rover should travel to your entered point and then return to the origin and face in the original direction. |  |  |