| Unit 7: The TI-RGB Array | Skill Builder 1: Light Them Up |  |  |  |
| --- | --- | --- | --- | --- |
| In this lesson, you will learn to control the sixteen color LEDs on the TI-RGB Array as a group and individually. | Objectives: |  |  |  |
|  | - Light up ALL LEDs and make them blink in unison - Use another loop to light up and turn off the LEDs one at a time |  |  |  |
| back BB ports The TI-RGB Array is a circuit board with 16 color LEDs and a controller chip and comes with a short 4-wire cable. It connects to the TI-Innovator using the cable wires that plug into breadboard (BB) ports on the TI-Innovator. Follow the wiring instructions on the back of the board to connect it to the TI-Innovator and then connect the TI-Innovator to your TI-Nspire CX. |  |  |  |  |
| Your first program will make all 16 LEDs on the TI-RGB Array blink 10 times. There are two instructions that control the Array: Send “CONNECT RGB” Send “SET RGB ALL r g b” or Send “SET RGB # r g b” # is an LED number (0 to 15) or eval(var or expression) r, g, b are in the range 0 to 255. |  |  |  |  |
| - Start a new program - we call it blink( ) - and add the statement Send “CONNECT RGB” by pressing menu > Hub > Send “CONNECT-Output > RGB and add the closing quotation mark: Send “CONNECT RGB” |  |  |  |  |
| - Add a For loop by pressing menu > Control > For…EndFor The loop parameters are: a variable (we used i) varying from 1 to 10 or an upper value of your choice: For I, 1, 10 But remember that when running the program you must wait for the program to end before doing anything else. There will be a better way of ending programs at any time in the next Skill Builder. |  |  |  |  |
| - In the loop body, add two Send statements: one to turn all the LEDs on and one to turn them all off. Follow each with a Wait statement: Send “SET RGB ALL 255 255 255” white Wait 0.5 Send “SET RGB ALL 0 0 0” off Wait 0.5 For the Send statements press menu>Hub>Send “SET… > RGB and complete the rest of both statements manually. Add the two Wait statement using menu>Hub>Wait and provide a time value. As with the COLOR LED on the Innovator and RV.COLOR on Rover there are three color values: red, green, and blue, each varying from 0 to 255. We used white LED’s, but you can create any color you like. |  |  |  |  |
| - Press ctrl-R to run the program and watch the LED’s on the RGB Array blink. Modify the program to display your favorite color. |  |  |  |  |
| Each of the sixteen LED’s can be controlled individually. Use the LED number (0 to 15) in place of ALL as in Send “SET RGB 5 255 0 0” turns LED #5 to red. If using a variable, then remember to use the eval( ) function found on the Hub menu. The next step illustrates this technique. |  |  |  |  |
| To control the LEDs one at a time, use an inner For loop and modify the Send commands to make use of the inner loop variable in place of ALL in the Send statements. - To ‘wrap’ the Send and Wait statements in another For loop, first select the four statements by placing the insertion cursor at the beginning of the block and press [shift] [downarrow]. Then… |  |  |  |  |
| - With the block selected, add the For loop by pressing menu > Control > For…EndFor. The selected text will be inside the new For and EndFor statements. - Complete the For statement. Note that the LEDs are numbered 0 to 15. - Change ALL to eval( j ) found on menu>Hub For i, 1, 5 use a smaller loop to go faster For j, 0, 15 Send “SET RGB eval( j ) 255 255 0” yellow Wait .1 make this smaller to go faster, too Send “SET RGB eval( j ) 0 0 0” Wait .1 make this smaller to go faster, too EndFor EndFor eval( is found on menu > HUB We also made the outer loop smaller and decreased the Wait value to 0.1 to speed things up. Run the program to see the effect. |  |  | a ‘moving’ light |  |