| Unit 7: The TI-RGB Array | Skill Builder 2: A Rainbow of Color |  |
| --- | --- | --- |
| In this lesson, you will generate random colors on each of the LEDs on the TI-RGB Array. | Objectives: |  |
|  | - Use getKey( ) to end a program - Use randInt( ) to light a random LED in a random color |  |
| We will use the random number generator randInt( ) to make random LEDs light up in random colors. |  |  |
| - Start a new program - we called it rainbow() - and add Send “CONNECT RGB” Using menu > HUB > Send “CONNECT-Output… and select RGB from the next menu. Type the closing quote and parenthesis. |  |  |
| - Add a While… EndWhile loop using menu > Control > While…EndWhile |  |  |
| - The loop will end when the [esc] key is pressed. The condition is: While getkey(0) ≠ “esc” ≠ is found on ctrl = type the quotes using [ctrl] [*] then the letters esc. |  |  |
| - In the loop body write four statements to assign random integers to four variables representing the LED number and the red, green, and blue values. led := randInt( 0, 15) red := randInt( 0, 255) green := randInt( 0, 255) blue := randInt( 0, 255) |  |  |
| - Add the Send statement to control one of the TI-RGB Array LEDs: Send “SET RGB eval(led) eval(red) eval(green) eval(blue)” and a Wait statement to slow the lights down: Wait .25 |  |  |
| - Press ctrl-R to run the program to see a wide variety of colors blinking. Press [esc] to stop the program. Notice that the LEDs are still lit even though the program has ended. Fix this by turning them off just after the EndWhile statement by adding the statement: EndWhile Send “SET RGB ALL 0 0 0” EndPrgm |  |  |