| Unit 6: Coordinates with Rover | Skill Builder 1: Drive and Plot |  |
| --- | --- | --- |
| In this lesson, you will learn about the Rover’s coordinate system and how to drive to a particular point represented by an ordered pair of numbers. | Objectives: |  |
|  | - Understand the Rover’s coordinate system and initial position and heading - Make the Rover move to a certain point on the coordinate plane - Plot Rover’s points on the TI-Nspire CX II screen |  |
| The Rover has a ‘built-in’ coordinate system just like a Cartesian graphing system. When you import ti_rover as rv, the Rover’s position on the coordinate grid is set to (0,0) and its heading is 0 degrees (pointing toward the positive x-axis, or ‘east’ on a map). |  |  |
| Teacher Tip: Rover also has a polar coordinate system! |  |  |
| In addition to Rover’s coordinate grid, we can use the on-screen coordinate system that is available from the ti_plotlib module which is also imported into the Rover Coding template for just this reason. If you use just these three statements from menu > TI PlotLib > Setup, you will get the screen to the right when you run: plt.window(-10,10,-7,7) plt.grid(1,1,"dashed") plt.axes("on") As we tell Rover to move around to points on the floor, we will also be plotting those points on the screen. |  |  |
| Teacher Tip: The Rover’s coordinate unit is the same as the driving unit: 10cm. The grid unit can be changed using rv.grid_m_unit(scale_value). The default scale_value is 0.1 m/unit (10cm). To make the grid unit one inch use the scale_value 0.0254. |  |  |
| - The first point to plot is Rover’s home position: (0, 0) using the statement plt.plot(x,y,”mark”) found on menu > TI PlotLib > Draw. Change the point (x, y) to (0, 0) and then select a “mark” to represent Rover from the provided list. If you run the program now, you will see a mark (we chose the dot ‘o’) at the origin in the center of the screen. Now, whenever Rover drives to a point on the floor, also plot the point on the screen. |  |  |
| - Depending on the space that you have, make Rover drive to a point in each of the four quadrants using rv.to_xy() found on menu > TI Rover > Drive. rv.to_xy(1,1) rv.to_xy(-1,1) rv.to_xy(-1,-1) rv.to_xy(1,-1) You do not have to use 1s and you don’t even have to use the same numbers. But you do have to make sure that Rover visits all four quadrants. Try your program now. Notice that Rover turns directly toward the position of the next point before moving there. |  |  |
| Teacher Tip: Rover might ‘wiggle’ a little when lining up for the next movement. This is Rover’s gyroscope fine-tuning the heading before driving. |  |  |
| - Add plt.plot(x,y,”mark”) statements to your program to plot the points on the screen after Rover reaches them. Try it now. Did the program perform as you expected? |  |  |
| Teacher Tip: Plotting on the screen is fast and easy using the ti_plotlib tools. |  |  |
| - Answer: No! The points on the screen are plotted almost immediately when the program runs, and it takes Rover some time to drive to all four points. How do you sync the plotting with the driving? |  |  |
| - Use the Rover function rv.wait_until_done() found on menu > TI Rover > Commands to pause the program while Rover is moving. You will need one of these functions for each driving point and order does matter. Try it now. Where will you place those wait functions? |  |  |
| Teacher Tip: The wait_until_done() functions belong after the rv_to_xy() and before the plt.plot() for each point: |  |  |
| - At the end of your program, send Rover to the ‘home’ position, (0, 0). Use the statement rv.to_angle(0, “degrees”) found on menu > TI Rover > Drive to have Rover point in the original direction (‘east’). It’s nice to put the toys away, right? |  |  |