| Command | Example | Behavior |
| --- | --- | --- |
| from module_name import * | from ti_hub import * | Imports all the functions in the ti_hub module for use in the program. The ti_hub module includes all the necessary additions needed for the Music project. |
| sound.tone(frequency,time) | sound.tone(440,1) | Plays a tone through the TI-Innovator Hub speaker. In the example, plays the frequency 440 hertz (Hz) for 1 second. |
| sleep(seconds) | sleep(1.5) | Pauses program for 1.5 seconds. |
| sound.note(“note”,time) | sound.tone(“C4”,.6) | Plays a note through the TI-Innovator Hub speaker. In the example, plays the note C4 (middle C, C in the 4th octave) for .6 seconds. The note is entered as a text string with capital letters for the note name. |
| # text comment | #Make sure your Hub is connected | # at the beginning of a line denotes a comment. Comments are a “best practice” by programmers to annotate their code. Comment statements are ignored when the program is run. In the TI-Nspire CXII Python editor, [ctrl]+[T] toggles the statement of the current cursor location from a comment to a statement that will be run. |

See TI-Innovator Hub Technology eGuide for more background on Hub-specific commands – Link

See TI-Nspire CXII Python Programming eGuide for more background on Python commands - Link